PX4 FIXED-WING

FLIGHT CONTROLLER STACK

ARCHITECTURE

TECS, NPFG, and the Six-Layer Cascade

from Mission Planning to Servo Output

Technical Reference Document

February 2026

# 1. Architecture Overview

PX4's fixed-wing autopilot is a six-layer cascaded control system that chains mission planning through position guidance, energy management, attitude tracking, angular rate regulation, and actuator allocation. All layers are connected by the uORB publish/subscribe middleware.

The two most distinctive subsystems are TECS (Total Energy Control System), which decouples altitude and airspeed control into total-energy and energy-balance channels, and NPFG (Nonlinear Path Following Guidance), a bearing-based lateral controller that replaced the older L1 algorithm and provides robust path tracking even in winds approaching airspeed.

Every module communicates via typed uORB topics, making the architecture fully modular: any block can be replaced at runtime without touching adjacent layers.

## 1.1 The Six-Layer Cascade

| Layer | Module | Function | Rate | Key Output |
| --- | --- | --- | --- | --- |
| 1 | Commander | Arming, mode select, failsafe | ~1 Hz | vehicle_status, vehicle_control_mode |
| 2 | Navigator | Mission, RTL, loiter, takeoff, land | ~1 Hz | position_setpoint_triplet |
| 3 | FW Mode Manager | Setpoint generation for all modes | ~50 Hz | FWLateralSetpoint, FWLongitudinalSetpoint |
| 4a | NPFG (Lateral) | Bearing-based path following | ~100 Hz | roll_body in att_setpoint |
| 4b | TECS (Longitudinal) | Energy-based alt/airspeed ctrl | ~100 Hz | pitch_body, thrust_body[0] |
| 5a | FW Att Control | Angle error to rate setpoints | 250 Hz | vehicle_rates_setpoint |
| 5b | FW Rate Control | PID+FF with airspeed scaling | 250 Hz | vehicle_torque/thrust_setpoint |
| 6 | Control Allocator | Effectiveness matrix inversion | 250 Hz | actuator_servos, actuator_motors |

## 1.2 uORB Signal Chain

| From → To | uORB Topic | Key Fields |
| --- | --- | --- |
| Commander → all | vehicle_status, vehicle_control_mode | nav_state, arming, loop flags |
| Navigator → Mode Mgr | position_setpoint_triplet | prev/current/next waypoints |
| Mode Mgr → Lat-Lon Ctrl | FWLateralSetpoint | course, heading, lat_accel |
| Mode Mgr → Lat-Lon Ctrl | FWLongitudinalSetpoint | altitude, height_rate, EAS |
| Lat-Lon Ctrl → Att Ctrl | vehicle_attitude_setpoint | roll_body, pitch_body, thrust |
| Att Ctrl → Rate Ctrl | vehicle_rates_setpoint | p, q, r setpoints (rad/s) |
| Rate Ctrl → Allocator | vehicle_torque_setpoint | roll, pitch, yaw torques |
| Allocator → Drivers | actuator_servos / actuator_motors | normalized commands |

# 2. TECS: Energy-Based Altitude and Airspeed Control

TECS, developed by A.A. Lambregts at Boeing in the early 1980s, solves the fundamental coupling problem in fixed-wing longitudinal control. Throttle and elevator both affect both airspeed and altitude simultaneously. TECS reframes the problem: throttle controls total energy (adding or removing energy), while pitch controls the distribution between kinetic energy (airspeed) and potential energy (altitude).

## 2.1 Energy Definitions

SPE = g · h                     (specific potential energy)

SKE = ½ · V_TAS²               (specific kinetic energy)

STE = SPE + SKE                  (specific total energy)

Taking time derivatives:

STE_rate = g·ḣ + V_TAS·V̇_TAS   (→ throttle channel)

SEB_rate = SPE_rate − SKE_rate    (→ pitch channel)

## 2.2 Outer Loops

alt_rate_sp = (alt_ref − alt) / FW_T_ALT_TC + FF·ḣ_ref

tas_rate_sp = (TAS_sp − TAS) / FW_T_TAS_TC

SPE_rate_sp = alt_rate_sp · g

SKE_rate_sp = V_TAS · tas_rate_sp

## 2.3 Throttle Channel (STE Rate Control)

STE_rate_error = STE_rate_sp − STE_rate_est

thr = thr_ff + STE_err·FW_T_THR_DAMPING·gain + thr_integ

Roll-to-throttle compensation: FW_T_RLL2THR · (load_factor − 1.0) is added to STE_rate_sp for banked turns.

## 2.4 Pitch Channel (SEB Rate Control)

SEB_rate_error = SEB_rate_sp − SEB_rate_est

θ_sp = SEB_err·FW_T_PTCH_DAMP·gain + FF·SEB_sp·gain + pitch_integ

## 2.5 Internal Submodules

The PX4 TECS library (src/lib/tecs/) comprises four classes: TECS (coordinator, 100 Hz), TECSControl (pitch/throttle PID), TECSAirspeedFilter (Kalman filter), and TECSAltitudeReferenceModel (jerk-limited trajectory). Throttle bypasses the attitude controller entirely.

## 2.6 Key TECS Parameters

| Parameter | Default | Role |
| --- | --- | --- |
| FW_T_ALT_TC | 5.0 s | Altitude error time constant (P-gain = 1/TC) |
| FW_T_TAS_TC | 5.0 s | Airspeed error time constant |
| FW_T_HRATE_FF | 0.3 | Height-rate feedforward |
| FW_T_THR_DAMPING | 0.05 | Throttle proportional gain on STE rate error |
| FW_T_THR_INTEG | 0.02 | Throttle integrator gain |
| FW_T_PTCH_DAMP | 0.1 | Pitch proportional gain on SEB rate error |
| FW_T_I_GAIN_PIT | 0.1 | Pitch integrator gain |
| FW_T_SEB_R_FF | 1.0 | SEB rate feedforward to pitch |
| FW_T_CLMB_MAX | 5.0 m/s | Max climb rate → sets STE_rate_max |
| FW_T_SINK_MAX | 5.0 m/s | Max descent rate |
| FW_T_RLL2THR | 15.0 | Roll-to-throttle load-factor compensation |
| FW_T_STE_R_TC | 0.4 s | STE rate estimate low-pass filter TC |

# 3. NPFG Lateral Guidance

NPFG, developed by Thomas Stastny at ETH Zurich / Auterion, operates on a bearing-based formulation rather than L1's geometric reference point. It computes a desired bearing angle from path geometry and track error, then uses a proportional controller on bearing error to generate lateral acceleration.

## 3.1 Control Law

a_lat = p_gain · bearing_error + a_lat_ff

p_gain = 2π / NPFG_PERIOD

time_const = NPFG_PERIOD / (2π · NPFG_DAMPING)

φ_sp = atan(a_lat / (g · cos(θ)))   constrained by FW_R_LIM

## 3.2 Wind Awareness

NPFG computes a bearing feasibility metric σ_feas based on β = |V_wind| / V_airspeed. When β < 1, tracking proceeds normally. When β ≥ 1 (wind exceeds airspeed), the controller smoothly transitions to turning into the wind to prevent runaway drift. It also features adaptive period adjustment for stability.

## 3.3 L1 Guidance (Legacy)

L1_dist = (1/π) · FW_L1_DAMPING · FW_L1_PERIOD · |V_ground|

a_lat = 2·V² / L1_dist · sin(η)

L1 has been deprecated in the main branch since v1.14.

# 4. Attitude and Rate Controllers

## 4.1 Attitude Controller — Time-Constant P-Loops

roll_rate_sp  = (φ_sp − φ) / FW_R_TC    constrained to ±FW_R_RMAX

pitch_rate_sp = (θ_sp − θ) / FW_P_TC   constrained to ±FW_P_RMAX

yaw_rate_sp   = (g / V) · tan(φ) · cos(θ)   (coordinated turn)

Turn compensation for pitch: pitch_rate_sp += |(g/V)·tan(φ)·sin(φ)|

FW_R_TC and FW_P_TC default to 0.5 s. Smaller values = more aggressive response. Yaw does not track a heading; it derives from coordinated-turn kinematics for zero-sideslip flight.

## 4.2 Rate Controller — PID+FF with Airspeed Scaling

τ = P·rate_error + I·∫(rate_error)dt − D·d(rate)/dt + FF·rate_sp

The D-term acts on feedback only (not error) to avoid derivative kick. The FF term compensates for aerodynamic damping and is typically the dominant term (defaults: FW_RR_FF=0.5, FW_PR_FF=0.5, FW_YR_FF=0.3).

### Airspeed Scaling

airspeed_scaling = FW_AIRSPD_TRIM / IAS

PI output × scale²     FF output × scale

This ensures consistent handling response from stall speed to max speed. Integrator anti-windup uses clamping plus a conditional factor: i_factor = max(0, 1 − (rate_error/400°)²)

## 4.3 Rate Controller Parameters

| Parameter | Default | Role |
| --- | --- | --- |
| FW_RR_P | 0.05 | Roll rate proportional gain |
| FW_RR_I | 0.01 | Roll rate integral gain |
| FW_RR_FF | 0.5 | Roll rate feedforward (dominant term) |
| FW_PR_P | 0.08 | Pitch rate proportional gain |
| FW_PR_I | 0.01 | Pitch rate integral gain |
| FW_PR_FF | 0.5 | Pitch rate feedforward |
| FW_YR_FF | 0.3 | Yaw rate feedforward |
| FW_AIRSPD_TRIM | 15 m/s | Trim airspeed for gain scaling |
| FW_RR_IMAX | 0.2 | Roll integrator limit |
| FW_PR_IMAX | 0.4 | Pitch integrator limit |

# 5. Control Allocator and Actuator Output

The modern control allocator (control_allocator, replacing legacy mixer files) uses an effectiveness matrix relating each actuator's contribution to each torque/thrust axis.

actuator_cmds = B⁻¹ · [τ_roll, τ_pitch, τ_yaw, T_x]ᵀ

PWM = PWM_MIN + (cmd + 1)/2 · (PWM_MAX − PWM_MIN)

For a standard fixed-wing: ailerons contribute roll torque, elevator contributes pitch torque, rudder contributes yaw torque, and the motor contributes forward thrust. For flying wings with elevons, the matrix captures the coupling. The allocator feeds back saturation status to the rate controller for integrator anti-windup.

# 6. Worked Signal-Flow Example

Consider a mission waypoint requiring a turn and climb:

1. Navigator publishes the waypoint triplet. FW Mode Manager generates lateral setpoint (desired course) and longitudinal setpoint (target altitude + cruise airspeed).

2. NPFG computes bearing error, applies proportional gain, converts lateral acceleration to a roll setpoint of 25°. Simultaneously, TECS sees the altitude error and commands throttle above trim while adjusting pitch upward.

3. Attitude controller computes: roll_rate_sp = (25° − current) / 0.5s; pitch_rate_sp + turn compensation; yaw_rate_sp = coordinated turn rate at 25° bank.

4. Rate controller applies PID+FF with airspeed scaling to produce torque demands. Control allocator maps to aileron, elevator, rudder deflections plus motor throttle.

The entire chain executes within a single control cycle. Attitude/rate loops run at 250 Hz, position-level loops at 50–100 Hz.

# 7. Key Architectural Design Choices

Energy decomposition: Throttle for total energy, pitch for energy balance eliminates the cross-coupling that plagues conventional altitude/speed PID loops.

Bearing-based guidance: NPFG provides explicit wind awareness with graceful degradation in extreme wind conditions, unlike L1's geometric approach.

Feedforward-dominant rate control: FF terms (0.3–0.5) account for aerodynamic damping directly, reducing reliance on feedback gains that can interact with structural modes.

Airspeed scaling: Inverse-airspeed gain scheduling ensures consistent handling response across the entire flight envelope without gain scheduling tables.

Modular uORB architecture: Each module subscribes only to outputs it needs. Post-v1.14 refactoring into fw_mode_manager → fw_lat_lon_control → fw_att_control → fw_rate_control → control_allocator provides clean interface boundaries.